Monitor.py

vk_human_detect(Interval):

Use visionkit to detect human,when people are recognized,get screenshot(image/jpg) from visionkit,then send sensor data and image to Slack

*you can use “for” to start a circle detect

while(True):

vk_human_detect(10)

*you can change

out=vk.vk_recog_face(picname)#no rectangle

into

out=vk.vk_box_face(picname)#have rectangle

to start a rectangle-have detection

the image got by vk.vk_box_face(picname) will be like this:

in slack,the data will be like this

with a rectangle!(vk_box_face)

Visionkit.py

def vk_box_stream(self,picname):

start a human detect stream in visionkit(lasting but no scrennshot)

def vk_box_face(self,picname):

start a human detect in visionkit(take a  scrennshot with rectangle and save to PI)

*if you want to use vk_box_face.py

in visionkit(~AIY-projects-python/src/examples/vision/)

you should put face_detection_rectangle.py here(like photo below)

(you can find face_detection_rectangle.py in our ラズベリーパイ/human_detect/AIY/)

and in visionkit(~AIY-projects-python/src/aiy/vision/)

you must replace annotator.py(~AIY-projects-python/src/aiy/vision/) like this:

visionkit's source code has a bug that can not screenshot(capture) while buffer_streaming

if you change code like this,you can make it and run face_detection_rectangle.py correctly

annotator.py changed code:

original_send_buffer = picamera.mmalobj.MMALPortPool.send_buffer

"""def silent_send_buffer(zelf, **kwargs):

try:

original_send_buffer(zelf, **kwargs)

except picamera.exc.PiCameraMMALError as error:

# Only silence MMAL_EAGAIN for our target instance.

our_target = self._overlay.renderer.inputs[0].pool == zelf

if not our_target or error.status != 14:

raise error"""

def silent_send_buffer(zelf, port , block=True,timeout=None):

try:

original_send_buffer(zelf,port,block,timeout)

except picamera.exc.PiCameraMMALError as error:

# Only silence MMAL_EAGAIN for our target instance.

our_target = self._overlay.renderer.inputs[0].pool == zelf

if not our_target or error.status != 14:

raise error

picamera.mmalobj.MMALPortPool.send_buffer = silent_send_buffer

or you can find annotator.py at our file (ラズベリーパイ/human_detect/AIY/)

def vk_recog_face(self,picname):

start a human detect in visionkit(take a  scrennshot without rectangle and save to PI)

def vk_human_detect(self,interval):

start a “for circle” human-detection(when detect a human ,scrennshot and save image to PI)

Slacktest.py/slack_photo.py

test slack function

Document /BME280

the dependence lib that can run Bme280 sensor to get environment data

Document /AIY

in this document,there are python codes that you should put them into Visionkit

So you can realize something new